M410agricultural drone

User Manual V1.4

2021.05

● M410 Airplane Introduction

● Features

● work efficiency

● configuration kit

● technical parameter

● M410 Airplane Introduction

95% of the whole machine is made of carbon fiber and magnesium-aluminum alloy material, modular design, durable and anti-fall, and the simple design is also convenient for later maintenance.

The medicine barrel and battery are quickly disassembled, and there are 2 quick disassembly water tanks as standard, and the next bucket of medicine can be changed in 10 seconds.

Folding the arm, the size is smaller after folding, and the empty weight does not include batteries and potions of 10KG, which is convenient for transit transportation.

● Features

Safety Obstacle Avoidance System

The forward obstacle avoidance radar is installed,

and the flight safety is upgraded again.

Terrain following ability

Equipped with a height detection radar, it is easy to deal

with slopes, mountains and fruit tree areas.

The standard H12 long-distance digital image control integrates an Android remote control. Equipped with a 5.5-inch high-brightness screen, you can see the picture clearly under the hot sun and operate easily. The remote control has a built-in 1000mAh battery with a battery life of 20 hours, which meets the needs of long-term high-intensity operations. There are LED lights on both sides of the high-definition camera, which is convenient for night work.

Airplane Modes and Features

Atti Star protection function, motor sequence detection function, direction detection function, dual water pump mode, drug break protection.

The battery has a built-in safety supervision system. The safety system will supervise the battery itself 24 hours a day. Once the voltage difference is too large, over-discharge, high temperature and other abnormal phenomena, the battery itself will sound an alarm, and the indicator light will flash in different forms. The abnormal state of the battery can be judged according to the abnormal state of the indicator light in the manual.

● work efficiency

Medicine chest capacity10L

Each sortie can complete 0.6 hectares，

1 battery can complete 2 sorties, i.e. 1.2 hectares，

It can complete 7-8 sorties per hour and complete 4.5 hectares of land，

8 hours a day, about 35 hectares of land can be completed

4 sets of batteries are used in turn and can be recycled, and the charging time of each battery is about 18 minutes.

●  configuration kit

● M410 Plant Protection Drone Rack *1

● 10L water tank*2

● JIYI flight controller*1

● Altitude Radar *1

● Forward Obstacle Avoidance Radar *1

● Flowmeter *1

● HD image transmission system*1

● 5.5 inch remote control *1

● X8 motor*4

● 30-inch propeller*4

● Spray system *1

● 2400W dual channel charger*1

● 12S 16AH intelligent lithium battery*4

● Spare parts kit*1

Spare Parts Kit Contains the following items:：

30 inch CW * 4 pieces             30 inch CCW * 4 pieces30         Nozzle assembly * 4 sets

Arm * 2

Signal line * 4                         Support frame assembly *1 set

XT 60 female * 2        XT 60 male head * 2                   front shell * 1

No matter how fast after-sales maintenance is, it is not as fast as the spare parts we donate by your side. Do it yourself and say goodbye to the long wait for aftermarket repairs.

technical parameter

- ●frame

- maximum wheelbase          990mm

- Dimensions           1725mm x 1725mm x 640mm(Arms unfolded, propellers unfolded）

- 640mm x 550mm x 640mm（Arm folded, paddle folded）

- ●power system

- Motor size                           21*20mm

- KV value                             100 KV

- maximum pulling force               15.3kg/rotor

- Motor assembly weight               1044g

- Maximum allowable voltage of ESC    52.2V

- ESC maximum allowable current (continuous)      80A

- Propeller Diameter*Pitch                          30*90 inch

- ●Sprinkler system

- Medicine box volume        rating: 10 L, full load: 10 L

- Spray width                 4-5 m (4 sprinklers, about 1.5-3 m from crops)

- Atomized particles are       110-230 μ m

- ●Forward Obstacle Avoidance Radar System

- Center frequency         24.125 GHz

- Product weight            80g

- Modulation mode         FMCW

- The ranging range is       1 ~ 25m

- Ranging accuracy          0.1 m

- Ranging resolution         0.6 m

- The output frequency is    40Hz

- Power consumption        3.0 W

- Operating voltage          9 ~ 28V

- Operating temperature     -20 ° C ~ + 85 ° C

- Protection level             IP67

- Output interface           CAN

- ●Anti-ground/fixed altitude radar system

- Center frequency          24.125 GHz

- Product weight           < 50g

- Modulation mode         FMCW

- There is no shelter under the use environment, and try to stay away from water mist

- Height measurement range is 0.3 ~ 15m (to the earth)

- Accuracy of height measurement is        0.05 m

- Output frequency                         50Hz

- Power consumption                       1.5 W @ 25 ° C

- Operating voltage                        6 ~ 28V

- Operating temperature                   -20 ° C ~ + 85 ° C

- Protection level              IP67

- Output interface             CAN

- * The effective working range of the radar module will vary depending on the data, position, shape, etc. of the target object.

- ●Flight parameters

- The weight of the whole machine (excluding batteries and potions) is    14.5 KG

- Standard takeoff weight              24.5 kg

- Maximum takeoff weight              29kg

- Hovering accuracy                   ± 0.6 m horizontally and ± 0.3 m vertically

- Smart battery                       12S/16000mah

- Hovering time                      25 min (16000 mAh takeoff weight 24.5 kg)

- 14 min (16000 mAh takeoff weight 24.5 kg)

- Maximum pitch angle               15

- Maximum operating flight speed        7 m/s

- Maximum horizontal flight speed        10 m/s (good signal)

- Maximum withstand wind speed         8 m/s

- The maximum take-off altitude is         3500 m

- Recommended working environment humidity         < 93%

- The recommended working environment temperature is         0 ℃ to 45 ℃

- ●Remote control

- Operating frequency from              2.4000 GHz to 2.4833 GHz

- The effective distance of signal is        20KM (no interference, no blocking)

- Display device 5.5 inch liquid crystal display screen

- Operating voltage              3.7 V

- Charging interface             MICRO-USB

- * Please comply with local regulations.

- ** The hovering time is measured near sea level with wind speed less than 3 m/s for reference only.

For detailed information of products, please visit the contact merchant * If the content is updated, it will not be notified separately.